LOGRING ver 1.4, August 21 1995 by Kai Risku

============================================

Logring is a small resident program (TSR) that watches one or several

modem ports for ring signals. When ring signals are detected, logring

will count the number of signals and then write the time, date and

number of signals to a file and optionally start flashing keyboard leds.

Logring can also monitor times and and durations of modem connections.

Logring will not interfere in any way with telecommunications on the

monitored port.

Installing LOGRING

==================

Usage: LOGRING [options] filename comport...

Filename should be the name of your logfile. The log texts will be

appended to the file if it already exists, otherwise the file is created.

Comport is the number of the com port of your modem. Specify 1 for

com1, 2 for com2 etc. If your com port is not recognized by dos, you

can specify the com port using the format 0xHHH, where HHH is the

base address of the port in hex.

Up to four port specifications separated by spaces can be entered.

If more than one port is specified, the logfile will also contain

information about which line the call came on (the first port

specified will be called line 1, the second port line 2, and so on).

You may delete or change the logfile at any time. Logring will simply

append to, or create the file when neccessary.

Options:   /M           log modem connections.

Using this option will cause Logring to monitor

your modem connections. The starting time and the

ending time as well as the total length of each

connection will be logged.

/F           flash led on keyboard.

Enabling this option will cause Logring to flash

the scroll lock led on the keyboard when a call

has been detected. The number of consecutive

flashes indicates the number of calls that have

been detected. Press and hold both left and right

shift for a couple of seconds to stop the flashing.

/T           display current time and date.

Using this option you can ensure that Logring is

able to get the time and date correctly on your

computer. This option may go away in the future.

/D           display extra info when installing.

This option will display the name of the logfile,

as well as the addresses of the com ports.

/U           uninstall program.

Removes the program from memory (if possible).

If you have loaded other memory resident program

(TSRs) after Logring, you might need to unload

those first (last in, first out) before Logring

can be removed from memory.

Options are NOT case sensitive, and several options can be combined

after the same switch character. One exception is that no more options

can be combined after an option that takes an argument. In this case you

must start with a new switch character.

Useful examples

===============

Example1: Your modem is on com2 and you want the text to be logged to

the file 'c:\temp\ring.log'. To do this you simply write:

LOGRING c:\temp\ring.log 2

Example2: You are monitoring two phone lines with modems connected to

com2 and com3 respectively. You want to log the call info to

the file 'd:\calls.lst' and also want to display some extra

info when installing the program. Because your dos does

not recognize com3, you specify the port address instead.

LOGRING /d d:\calls.lst 2 0x3e8

Example3: Your modem on com4 is accepting modem connections, and you

now wish to log exactly when and how long these connections

are. To make your life even harder, your dos does not

recognize com4 because you have no com3. You would also like

the Scroll Lock led to flash to indicate when there has been

incoming phone calls.

LOGRING /mf c:\modem.log 0x2e8

Example4: Logring is already installed and resident in memory, but you

have forgotten the parameters used and wish to display them.

LOGRING /d

Technical info

==============

Logring works by polling the Ring Indicator flag of the status register

in the com port. This means you must have a modem with a working ring

indicator signal connected to your computer (either an internal modem or

an external modem connected with a suitable cable).

Modem connections are monitored by reading the DCD signal (Carrier Detect)

as supplied by the modem. As with the Ring Indicator signal, your

modem cable must have DCD connected for this feature to work.

Polling the status register should not interfere with any other

communication, FAX TSR or similar program.

Common questions

================

I question I quite often get via email, is if/when Logring will support

Caller-ID. I don't know! As is it now, I do not have a CID-compatible

modem myself, the CID service is not publicly available here in Finland

yet and capturing the CID info from the modem would require significant

changes in Logring. The last point is motivated with the fact that

checking the RI bit in the status register is extremely simple and can

be done regardless of speeds or other settings. Receiving the CID data

from the modem would need extra interrupts (that will be stolen as soon

as you run a terminal program) as well as modem and serial port

initializations.

I have seen programs similar to Logring that works only under Windows

but should be able to receive CID information. Grep the index of your

nearest Simtel-mirror for details.

Logring is not quite compatible with Windows multitasking (ha!), but

incoming calls while you are running Win are buffered in memory and

written to the logfile when you exit. When running DOS prompts under

Windows, you will get warnings about your modem port being in use unless

you set the Device Contention in Control Panel/Enhanced to 'Never Warn'.

Logring is 100% hand-written assembler. Think again before you volunteer

to port Logring to Linux.. ;)

Revision history

================

Version 1.4:    Dropped support for different clock chips and made

Logring use only the bios clock to get the time and date.

This should work on most compatibles (although some

versions of DOS can cause trouble). Added option /f to

flash led on keyboard to signal incoming calls. Improved

ability to write to the logfile when other programs are

active.

Version 1.3:    Added ability to log time and length of modem connections.

Changed the looks of the logfile to a more suitable format.

Version 1.2:    Option /d can now show info for already installed program.

Created tiny environment so "owner" will show correctly

for some memory map programs. Added support for VICTOR clock

card and BIOS timer. Options can now begin with either

'/' or '-'. Hex address of ports can now be used instead

of port number. Clock type and optional address can now be

overridden on the command line. Switch for displaying

current time and date added (good for trying different

clock types).

Version 1.1:    Added option to remove program from memory. Added support

for XT clock cards. Several com ports can be monitored.

Time of incoming call now contains seconds as well.

Option to display extra information added.

Version 1.0:    Initial release of program.

#include <std_disclaimer.h>:

============================

Copyright (c) 1992-1995 Kai Risku

LOGRING is offered to you "AS IS" without any warranty. This software

has been thoroughly tested but no guarantee is given that it will work

on every computer. The copyright owner may not be held liable for any

damages, direct or consequential, which may result from the the use of

this program.

This program is freely distributable. You may use the software and

share it with all your friends (and foes) as long as the program is

supplied in its original, unmodified form, which includes this

documentation. This program must not be distributed for profit.

If you have any questions, ideas, suggestions for improvements or if

you are interested in the source then feel free to contact me...

If this program is worth anything to you, I would appreciate any token

of gratitude you can think of. User support is highly dependent on the

subjective value of your appreciation in a not necessarily causal

manner, as well as the amount of temporal slack in author-space.

The latest version of Logring, as well as some beta versions of this

and other software I have written, can be found via my WWW home page.

Snailmail:      Kai Risku         Email (internet):  Kai.Risku@hut.fi

Kronvik           WWW:     http://www.hut.fi/~krisku/

65410 Sundom

Finland